Gentlemen,

Doug and I have been discussing one of my action items from the November ISAC

meeting.  The issue has to do with the meaning of Item 9 of Paragraph 3 of

Section 7.3.2.2 (page 7-11).

In order to guage the scope of the problem, I'd like to take a survey.  Please

examine the attached package and tell me which constants are legal and which

are illegal.  Please provide me with any LRM references (the actual text is

unnecessary--this is not an IR!) that you deem appropriate.

Thanks in advance,

--Paul

-------------------------------------------------------------------------------

package P is

type A1 is array (0 to 0, 31 downto 0) of Bit;

constant A1_1: A1 := (0      => X"00000000");

constant A1_2: A1 := (others => X"00000000");

constant A1_3: A1 := (0      => (others => '0'));

constant A1_4: A1 := (others => (others => '0'));

type A2 is array (0 to 0) of Bit_Vector (31 downto 0);

constant A2_1: A2 := (0      => X"00000000");

constant A2_2: A2 := (others => X"00000000");

constant A2_3: A2 := (0      => (others => '0'));

constant A2_4: A2 := (others => (others => '0'));

type R1 is record

F: Bit_Vector (31 downto 0);

end record;

constant R1_1: R1 := (F      => X"00000000");

constant R1_2: R1 := (others => X"00000000");

constant R1_3: R1 := (F      => (others => '0'));

constant R1_4: R1 := (others => (others => '0'));

type R2 is record

F: R1;

end record;

constant R2_1: R2 := (F      => (F      => X"00000000"));

constant R2_2: R2 := (others => (F      => X"00000000"));

constant R2_3: R2 := (F      => (others => X"00000000"));

constant R2_4: R2 := (others => (others => X"00000000"));

end P;