- VHDL SceMiMacros package

- (Informative)

The following package can be used to supply SCE-MI macro component declarations to an application. Compile this package into the library SceMi and include it in the application code as:

library SceMi;

use SceMi.SceMiMacros.all;

Here is the source code for the package:

library ieee;

use ieee.std_logic_1164.all;

package SceMiMacros is

component SceMiMessageInPort

generic( PortWidth: natural );

port(

ReceiveReady  : in  std_logic;

TransmitReady : out std_logic;

Message       : out std_logic_vector( PortWidth-1 downto 0 ) );

end component;

component SceMiMessageOutPort

generic( PortWidth: natural; PortPriority: natural:=10 );

port(

TransmitReady : in  std_logic;

ReceiveReady  : out std_logic;

Message       : in  std_logic_vector( PortWidth-1 downto 0 ) );

end component;

component SceMiClockPort

generic(

ClockNum         : natural := 1;

RatioNumerator   : natural := 1;

RatioDenominator : natural := 1;

DutyHi           : natural := 0;

DutyLo           : natural := 100;

Phase            : natural := 0;

ResetCycles      : natural := 8 );

port(

Cclock : out std_logic;

Creset : out std_logic );

end component;

component SceMiClockControl

generic( ClockNum: natural := 1 );

port(

Uclock,

Ureset                : out std_logic;

ReadyForCclock        : in  std_logic;

CclockEnabled         : out std_logic;

ReadyForCclockNegEdge : in  std_logic;

CclockNegEdgeEnabled  : out std_logic );

end component;

end SceMiMacros;