- Multi-clock hardware side interface example

(Informative)

Figure XX shows the top level structure of a simple multi-clock, multi-transactor example.

- Multi-clock, multi-transactor example

This design demonstrates the following points.

Three ClockPort instances define clocks named cclock, cclock2_1, and cclock4_1.

- Because no parameters are given with the SceMiClockPort instance cclock, all default parameters are used. This means cclock has a ClockNum=1, a clock ratio of 1/1, a don’t care duty cycle, a phase shift of 0, and the controlled reset it supplies has an active duration of eight controlled clock cycles.

- The cclock2_1 instance of SceMiClockPort overrides the first three parameters and leaves the rest at their default values. This means cclock2_1 has a ClockNum=2, a clock ratio of 2/1 (i.e., a “divide-by-2” clock), a duty cycle of 50%, a phase shift of 0, and an eight clock-cycle reset duration.

- The cclock4_1 instance of SceMiClockPort has a ClockNum=3, a clock ratio of 4/1 (i.e., a “divide-by-4” clock), a duty cycle of 75%, a phase shift of 30% of the clock period, and an eight clock- cycle reset duration.

- The TxTransactor transactor model, named Bridge.u1, controls clocks cclock and cclock2_1 since its SceMiClockControl macro instances have ClockNum=1 and 
ClockNum=2, respectively.

- This TxTransactor model interfaces to a message input port called p1 which is parametrized to a bit- width of 64.

- The RxTransactor transactor model, named Bridge.u2, controls clock cclock4_1 since its SceMiClockControl macro instance has ClockNum=3.

- This RxTransactor model interfaces to a message input port called p1 which is parametrized to a bit- width of 128.

The following listing shows some of the VHDL source code for the above schematic.

library ieee;

use ieee.std_logic_1164.all;

library SceMi;

use SceMi.SceMiMacros.all;

entity Bridge is end;

architecture Structural of Bridge is

component TxTransactor is

port(

DutInCtrl:  out std_logic;

DutInData:  out std_logic_vector(31 downto 0);

DutOutCtrl: in  std_logic;

DutOutData: in  std_logic_vector(31 downto 0) );

end component TxTransactor;

component TxDUT is

port(

DutInCtrl:  in  std_logic;

DutInData:  in  std_logic_vector(31 downto 0);

DutOutCtrl: out std_logic;

DutOutData: out std_logic_vector(31 downto 0);

Clk, Rst, ClkDiv2: in std_logic );

end component TxDUT;

component RxTransactor is

port(

DutInCtrl:  out std_logic;

DutInData:  out std_logic_vector(31 downto 0);

DutOutCtrl: in  std_logic;

DutOutData: in  std_logic_vector(31 downto 0) );

end component RxTransactor;

component RxDUT is

port(

DutInCtrl:  in  std_logic;

DutInData:  in  std_logic_vector(31 downto 0);

DutOutCtrl: out std_logic;

DutOutData: out std_logic_vector(31 downto 0);

Clk, Rst: in std_logic );

end component RxDUT;

signal txDutInCtrl, txDutOutCtrl: std_logic;

signal txDutInData, txDutOutData: std_logic_vector(31 downto 0);

signal rxDutInCtrl, rxDutOutCtrl: std_logic;

signal rxDutInData, rxDutOutData: std_logic_vector(31 downto 0);

signal cclock, creset, clkDivideBy2, clkDivideBy4 
            cresetDivideBy4: std_logic;

begin

u1: TxTransactor port map( txDutInCtrl, txDutInData, txDutOutCtrl,
                               txDutOutData );

d1: TxDUT        port map( txDutInCtrl, txDutInData, txDutOutCtrl,
                               txDutOutData, cclock, creset, clkDivideBy2 );

cclock:    SceMiClockPort port map( cclock, creset );

cclock2_1: SceMiClockPort

generic map( 2, 2, 1, 50, 50, 0, 8 )

port map( clkDivideBy2, open );

u2: RxTransactor port map( txDutInCtrl, txDutInData, txDutOutCtrl,
                               txDutOutData );

d2: RxDUT        port map( txDutInCtrl, txDutInData, txDutOutCtrl,
                                 txDutOutData, clkDivideBy4, cresetDivideBy4 );

cclock4_1: SceMiClockPort

generic map( 3, 4, 1, 75, 25, 30, 8 )

port map( clkDivideBy2, open );

end;

library ieee;

use ieee.std_logic_1164.all;

library SceMi;

use SceMi.SceMiMacros.all;

entity TxTransactor is

port(

DutInCtrl:  out std_logic;

DutInData:  out std_logic_vector(31 downto 0);

DutOutCtrl: in  std_logic;

DutOutData: in  std_logic_vector(31 downto 0) );

end;

architecture Structural of TxTransactor is

component TxTransactorCore is

port(

TxRdyIn: in std_logic;           RxRdyIn: out std_logic;

Message: in std_logic(63 downto 0);

DutInCtrl:  out std_logic;

DutInData:  out std_logic_vector(31 downto 0);

DutOutCtrl: in  std_logic;

DutOutData: in  std_logic_vector(31 downto 0) );

Uclk, Rst: in std_logic;

ReadyForCclock:     in std_logic;

CclockEnabled:     out std_logic;

ReadyForCclockDiv2: in std_logic;

CclockEnabledDiv2: out std_logic;

end component TxTransactor;

signal transmitReady, receiveReady: std_logic;

signal message: std_logic_vector(63 downto 0);

signal uclock, ureset: std_logic;

signal readyForCclock, cclockEnabled: std_logic;

signal readyForCclockDiv2, cclockEnabledDiv2;

begin

t1: TxTransactorCore port map(

transmitReady, receiveReady, message,

DutInCtrl, DutInData, DutOutCtrl, DutOutData,

uclock, ureset,

readyForCclock, cclockEnabled, readyForCclockDiv2,
                             cclockEnabledDiv2 );

p1: SceMiMessageInputPort

generic map( 64 )

port map( transmitReady, receiveReady, message );

c1: SceMiClockControl

port map( uclock, ureset, readyForCclock, cclockEnabled, 
                     ‘1’, open );

c2: SceMiClockControl

generic map( 2 )

port map( open, open, readyForCclockDiv2, cclockEnabledDiv2,
                     ‘1’, open );

end;