- Sample Header files for the SCE-MI

(informative)

The ANSI-C file should be used without modification. For the C++ header, extensions are allowable but no modifications can be made to any of the contents that are provided.

  - C++

//

// Copyright © 2003-2005 by Accellera

// scemi.h - SCE-MI C++ Interface

//

#ifndef INCLUDED_SCEMI

#define INCLUDED_SCEMI

class SceMiParameters;

class SceMiMessageData;

class SceMiMessageInPortProxy;

class SceMiMessageOutPortProxy;

#define SCEMI_MAJOR_VERSION 1

#define SCEMI_MINOR_VERSION 1

#define SCEMI_PATCH_VERSION 0

#define SCEMI_VERSION_STRING "1.1.0"

/* 32 bit unsigned word type for building and reading messages */

typedef unsigned int SceMiU32;

/* 64 bit unsigned word used for CycleStamps */

typedef unsigned long long SceMiU64;

extern "C" {

typedef int (*SceMiServiceLoopHandler)(void* context, int pending);

};

/*

* struct SceMiEC - SceMi Error Context

*/

typedef enum {

SceMiOK,

SceMiError

} SceMiErrorType;

typedef struct {

const char* Culprit;   /* The offending function */

const char* Message;   /* Descriptive message describing problem */

SceMiErrorType Type;   /* Error code describing the nature of the error */

int Id;                /* A code to uniquely identify each error */

} SceMiEC;

extern "C" {

typedef void (*SceMiErrorHandler)(void* context, SceMiEC* ec);

};

/*

* struct SceMiIC - SceMi Informational Message Context

*/

typedef enum {

SceMiInfo,

SceMiWarning,

SceMiNonFatalError

} SceMiInfoType;

typedef struct {

const char* Originator;

const char* Message;

SceMiInfoType Type;

int Id;

} SceMiIC;

extern "C" {

typedef void (*SceMiInfoHandler)(void* context, SceMiIC* ic);

};

/*

* struct SceMiMessageInPortBinding

*

* Description

* -----------

* This structure defines a tray of callback functions that support

* propagation of message input readiness back to the software.

*

* If an input ready callback is registered (optionally) on a given

* input port, the port will dispatch the callback whenever becomes

* ready for more input.

*

* Note: All callbacks must take their data and return promptly as they

* are called possibly deep down in a non-preemptive thread.  Typically,

* the callback might to some minor manipulation to the context object

* then return and let a suspended thread resume and do the main process¬ing

* of the received transaction.

*/

extern "C" {

typedef struct {

/*

* This is the user's context object pointer.

* The application is free to use this pointer for any purposes it

* wishes.  Neither the class SceMi nor class MessageInputPortProxy do

* anything with this pointer other than store it and pass it when

* calling functions.

*/

void* Context;

/*

* Receive a response transaction.  This function is called when data

* from the message output port arrives.  This callback acts as a proxy

* for the message output port of the transactor.

*/

void (*IsReady)(

void* context);

/*

* This function is called from the MessageInputPortProxy destructor

* to notify the user code that the reference to the 'context' pointer

* has been deleted.

*/

int (*Close)(

void* context);

} SceMiMessageInPortBinding;

};

/*

* struct SceMiMessageOutPortBinding

*

* Description

* -----------

* This structure defines a tray of callback functions are passed to the class

* SceMi when the application model binds to a message output port proxy and

* which are called on message receipt and close notification.  It is the means

* by which the MessageOutputPort forwards received transactions to the C model.

*

* Note: All callbacks must take their data and return promptly as they

* are called possibly deep down in a non-preemptive thread.  Typically,

* the callback might to some minor manipulation to the context object

* then return and let a suspended thread resume and do the main process¬ing

* of the received transaction.

*

* Additionally, the message data passed into the receive callback is

* not guaranteed to remain the same once the callback returns.  All

* data therein then must be processed while inside the callback.

*/

extern "C" {

typedef struct {

/*

* This is the user's context object pointer.

* The application is free to use this pointer for any purposes it

* wishes.  Neither the class SceMi nor class SceMiMessageOutPortProxy do

* anything with this pointer other than store it and pass it when

* calling callback functions Receive and Close.

*/

void* Context;

/*

* Receive a response transaction.  This function is called when data

* from the message output port arrives.  This callback acts as a proxy

* for the message output port of the transactor.

*/

void (*Receive)(

void* context,

const SceMiMessageData* data);

/*

* This function is called from the MessageOutputPortProxy destructor

* to notify the user code that the reference to the 'context' pointer

* has been deleted.

*/

int (*Close)(

void* context);

} SceMiMessageOutPortBinding;

};

class SceMiParameters {

public:

// CREATORS

//

// This constructor initializes some parameters from the

// parameters file in the config directory, and some other

// parameters directly from the config file.

//

SceMiParameters(

const char* paramsfile,

SceMiEC* ec = 0);

~SceMiParameters();

// ACCESSORS

//

// This accessor returns the number of instances of objects of

// the specified objectKind name.

//

unsigned int NumberOfObjects(

const char* objectKind,    // Input: Object kind name.

SceMiEC* ec = 0) const; // Input/Output: Error status.

//

// These accessors return an integer or string attribute values of the

// given object kind.  It is considered an error if the index > number

// returned by ::NumberOfObjects() or the objectKind and attributeName

// arguments are unrecognized.

//

int AttributeIntegerValue(

const char* objectKind,    // Input: Object kind name.

unsigned int index,        // Input: Index of object instance.

const char* attributeName, // Input: Name of attribute being read.

SceMiEC* ec = 0) const;    // Input/Output: Error status.

const char* AttributeStringValue(

const char* objectKind,    // Input: Object kind name.

unsigned int index,        // Input: Index of object instance.

const char* attributeName, // Input: Name of attribute being read.

SceMiEC* ec = 0) const;    // Input/Output: Error status.

// MANIPULATORS

//

// These manipulators override an integer or string attribute values of the

// given object kind.  It is considered an error if the index > number

// returned by ::NumberOfObjects(). or the objectKind and attribute¬Name

// arguments are unrecognized.

//

void OverrideAttributeIntegerValue(

const char* objectKind,    // Input: Object kind name.

unsigned int index,        // Input: Index of object instance.

const char* attributeName, // Input: Name of attribute being read.

int value,                 // Input: New integer value of attribute.

SceMiEC* ec = 0);          // Input/Output: Error status.

void OverrideAttributeStringValue(

const char* objectKind,    // Input: Object kind name.

unsigned int index,        // Input: Index of object instance.

const char* attributeName, // Input: Name of attribute being read.

const char* value,         // Input: New string value of attribute.

SceMiEC* ec = 0);          // Input/Output: Error status.

};

//

// class SceMiMessageInPortProxy

//

// Description

// -----------

// The class SceMiMessageInPortProxy presents a C++ proxy for a transactor

// message input port.  The input channel to that transactor is repre¬sented

// by the Send() method.

//

class SceMiMessageInPortProxy {

public:

// ACCESSORS

const char* TransactorName() const;

const char* PortName() const;

unsigned int PortWidth() const;

//

// This method sends message to the transactor input port.

//

void Send(

const SceMiMessageData &data, // Message payload to be sent.

SceMiEC* ec = 0);

//

// Replace port binding.

// The binding argument represents a callback function and context

// pointer tray (see comments in scemicommontypes.h for struct

// SceMiMessageInPortBinding).

//

void ReplaceBinding(

const SceMiMessageInPortBinding* binding = 0,

SceMiEC* ec = 0);

};

//

// class SceMiMessageOutPortProxy

//

// Description

// -----------

// The class SceMiMessageOutPortProxy presents a C++ proxy for a transac¬tor

// message output port.

//

class SceMiMessageOutPortProxy {

public:

// ACCESSORS

const char* TransactorName() const;

const char* PortName() const;

unsigned int PortWidth() const;

//

// Replace port binding.

// The binding argument represents a callback function and context

// pointer tray (see comments in scemicommontypes.h for struct

// SceMiMessageOutPortBinding).

//

void ReplaceBinding(

const SceMiMessageOutPortBinding* binding = 0,

SceMiEC* ec = 0);

};

//

// class SceMiMessageData

//

// Description

// -----------

// The class SceMiMessageData represents a fixed length array of data which

// is transferred between models.

//

class SceMiMessageData {

public:

// CREATORS

//

// Constructor: The message in port proxy for which

// this message data object must be suitably sized.

//

SceMiMessageData(

const SceMiMessageInPortProxy& messageInPortProxy,

SceMiEC* ec = 0);

~SceMiMessageData();

// Return size of vector in bits

unsigned int WidthInBits() const;

// Return size of array in 32 bit words.

unsigned int WidthInWords() const;

void Set( unsigned i, SceMiU32 word, SceMiEC* ec = 0);

void SetBit( unsigned i, int bit, SceMiEC* ec = 0);

void SetBitRange(

unsigned int i, unsigned int range, SceMiU32 bits, SceMiEC* ec = 0);

SceMiU32 Get( unsigned i, SceMiEC* ec = 0) const;

int GetBit( unsigned i, SceMiEC* ec = 0) const;

SceMiU32 GetBitRange(

unsigned int i, unsigned int range, SceMiEC* ec = 0) const;

SceMiU64 CycleStamp() const;

};

//

// class SceMi

//

// Description

// -----------

// This file defines the public interface to class SceMi.

//

class SceMi {

public:

//

// Check version string against supported versions.

// Returns -1 if passed string not supported.

// Returns interface version # if it is supported.

// This interface version # can be passed to SceMi::Init().

//

static int Version(

const char* versionString);

//

// This function wraps constructor of class SceMi.  If an instance

// of class SceMi has been established on a prior call to the

// SceMi::Init() function, that pointer is returned since a single

// instance of class SceMi is reusable among all C models.

// Returns NULL if error occurred, check ec for status or register

// an error callback.

//

// The caller is required to pass in the version of SceMi it is

// expecting to work with.  Call SceMi::Version to convert a version

// string to an integer suitable for this version's "version" argu¬ment.

//

// The caller is also expected to have instantiated a SceMiParameters

// object, and pass a pointer to that object into this function.

//

static SceMi*

Init(

int version,

const SceMiParameters* parameters,

SceMiEC* ec = 0);

//

// Shut down the SCEMI interface.

//

static void

Shutdown(

SceMi* mct,

SceMiEC* ec = 0);

//

// Create proxy for message input port.

//

// Pass in the instance name in the bridge netlist of

// the transactor and port to which binding is requested.

//

// The binding argument is a callback function and context

// pointer tray.  For more details, see the comments in

// scemicommontypes.h by struct SceMiMessageInPortBinding.

//

SceMiMessageInPortProxy*

BindMessageInPort(

const char* transactorName,

const char* portName,

const SceMiMessageInPortBinding* binding = 0,

SceMiEC* ec = 0);

//

// Create proxy for message output port.

//

// Pass in the instance name in the bridge netlist of

// the transactor and port to which binding is requested.

//

// The binding argument is a callback function and context

// pointer tray.  For more details, see the comments in

// scemicommontypes.h by struct SceMiMessageOutPortBinding.

//

SceMiMessageOutPortProxy*

BindMessageOutPort(

const char* transactorName,

const char* portName,

const SceMiMessageOutPortBinding* binding = 0,

SceMiEC* ec = 0);

//

// Service arriving transactions from the portal.

// Messages enqueued by SceMiMessageOutPortProxy methods, or which are

// are from output transactions that pending dispatch to the

// SceMiMessageOutPortProxy callbacks, may not be handled until

// ServiceLoop() is called.  This function returns the # of output

// messages that were dispatched.

//

// Regarding the service loop handler (aka "g function"):

// If g is NULL, check for transfers to be performed and

// dispatch them returning immediately afterwards.  If g is

// non-NULL, enter into a loop of performing transfers and

// calling 'g'.  When 'g' returns 0 return from the loop.

// When 'g' is called, an indication of whether there is at

// least 1 message pending will be made with the 'pending' flag.

//

// The user context object pointer is uninterpreted by

// ServiceLoop() and is passed straight to the 'g' function.

//

int

ServiceLoop(

SceMiServiceLoopHandler g = 0,

void* context = 0,

SceMiEC* ec = 0);

//

// Register an error handler which is called in the event

// that an error occurs.  If no handler is registered, the

// default error handler is called.

//

static void

RegisterErrorHandler(

SceMiErrorHandler errorHandler,

void* context);

//

// Register an info handler which is called in the event

// that a text message needs to be issued.  If no handler

// is registered, the message is printed to stdout in

// Ikos message format.

//

static void

RegisterInfoHandler(

SceMiInfoHandler infoHandler,

void* context);

};

#endif

  - ANSI-C

/*

* scemi.h

*

* Copyright © 2003-2005 by Accellera

* This file is the header file for the SCEMI C API.

*/

#ifndef INCLUDED_SCEMI

#define INCLUDED_SCEMI

typedef void SceMi;

typedef void SceMiParameters;

typedef void SceMiMessageData;

typedef void SceMiMessageInPortProxy;

typedef void SceMiMessageOutPortProxy;

#define SCEMI_MAJOR_VERSION 1

#define SCEMI_MINOR_VERSION 1

#define SCEMI_PATCH_VERSION 0

#define SCEMI_VERSION_STRING "1.1.0"

/* 32 bit unsigned word type for building and reading messages */

typedef unsigned int SceMiU32;

/* 64 bit unsigned word used for CycleStamps */

typedef unsigned long long SceMiU64;

typedef int (*SceMiServiceLoopHandler)(void* context, int pending);

/*

* struct SceMiEC - SceMi Error Context

*/

typedef enum {

SceMiOK,

SceMiError

} SceMiErrorType;

typedef struct {

const char* Culprit;   /* The offending function */

const char* Message;   /* Descriptive message describing problem */

SceMiErrorType Type;   /* Error code describing the nature of the error */

int Id;                /* A code to uniquely identify each error */

} SceMiEC;

typedef void (*SceMiErrorHandler)(void* context, SceMiEC* ec);

/*

* struct SceMiIC - SceMi Informational Message Context

*/

typedef enum {

SceMiInfo,

SceMiWarning,

SceMiNonFatalError

} SceMiInfoType;

typedef struct {

const char* Originator;

const char* Message;

SceMiInfoType Type;

int Id;

} SceMiIC;

typedef void (*SceMiInfoHandler)(void* context, SceMiIC* ic);

/*

* struct SceMiMessageInPortBinding

*

* Description

* -----------

* This structure defines a tray of callback functions that support

* propagation of message input readiness back to the software.

*

* If an input ready callback is registered (optionally) on a given

* input port, the port will dispatch the callback whenever becomes

* ready for more input.

*

* Note: All callbacks must take their data and return promptly as they

* are called possibly deep down in a non-preemptive thread.  Typically,

* the callback might to some minor manipulation to the context object

* then return and let a suspended thread resume and do the main process¬ing

* of the received transaction.

*/

typedef struct {

/*

* This is the user's context object pointer.

* The application is free to use this pointer for any purposes it

* wishes.  Neither the class SceMi nor class MessageInputPortProxy do

* anything with this pointer other than store it and pass it when

* calling functions.

*/

void* Context;

/*

* Receive a response transaction.  This function is called when data

* from the message output port arrives.  This callback acts as a proxy

* for the message output port of the transactor.

*/

void (*IsReady)(

void* context);

/*

* This function is called from the MessageInputPortProxy destructor

* to notify the user code that the reference to the 'context' pointer

* has been deleted.

*/

int (*Close)(

void* context);

} SceMiMessageInPortBinding;

/*

* struct SceMiMessageOutPortBinding

*

* Description

* -----------

* This structure defines a tray of callback functions are passed to the class

* SceMi when the application model binds to a message output port proxy and

* which are called on message receipt and close notification.  It is the means

* by which the MessageOutputPort forwards received transactions to the C model.

*

* Note: All callbacks must take their data and return promptly as they

* are called possibly deep down in a non-preemptive thread.  Typically,

* the callback might to some minor manipulation to the context object

* then return and let a suspended thread resume and do the main process¬ing

* of the received transaction.

*

* Additionally, the message data passed into the receive callback is

* not guaranteed to remain the same once the callback returns.  All

* data therein then must be processed while inside the callback.

*/

typedef struct {

/*

* This is the user's context object pointer.

* The application is free to use this pointer for any purposes it

* wishes.  Neither the class SceMi nor class SceMiMessageOutPortProxy do

* anything with this pointer other than store it and pass it when

* calling callback functions Receive and Close.

*/

void* Context;

/*

* Receive a response transaction.  This function is called when data

* from the message output port arrives.  This callback acts as a proxy

* for the message output port of the transactor.

*/

void (*Receive)(

void* context,

const SceMiMessageData* data);

/*

* This function is called from the MessageOutputPortProxy destructor

* to notify the user code that the reference to the 'context' pointer

* has been deleted.

*/

int (*Close)(

void* context);

} SceMiMessageOutPortBinding;

/*

* Register an error handler which is called in the event

* that an error occurs.  If no handler is registered, the

* default error handler is called.  The errorHandler will

* pass back the 'context' object registered by the user

* when making this function call.  The system makes no

* assumptions about the 'context' pointer and will not

* modify it.

*/

void

SceMiRegisterErrorHandler(

SceMiErrorHandler errorHandler,

void* context);

/*

* Register an info handler which is called in the event

* that an informational text message needs to be printed.

* If no handler is registered, the message is printed to stdout.

*/

void SceMiRegisterInfoHandler(

SceMiInfoHandler infoHandler,

void* context );

/*

* Check version string against supported versions.

* Return -1 if passed string not supported.

* Return interface version # if it is supported. This interface

* version # can be passed to the SceMiInit() function.

*/

int

SceMiVersion(

const char* versionString);

/*

* This function wraps constructor of class SceMi.  If an instance

* of class SceMi has been established on a prior call to the

* the SceMiInit() function, that pointer is returned since a single

* instance of class SceMi is reusable among all C models.

*

* The caller must provide the interface version # it is expecting

* to work with.  If the caller requests an unsupported version,

* an error is returned.

*

* The caller must also provide a pointer to a filled-in SceMiParameters

* struct that contains global interface specification parameters.

*

* Returns NULL if error occurred, check ec for status or register

* an error callback.

*/

SceMi*

SceMiInit(

int version,

const SceMiParameters* parameters,

SceMiEC* ec);

/*

* Shut down the specified SCEMI interface.

*/

void

SceMiShutdown(

SceMi* mctHandle,

SceMiEC* ec);

/*

* Create proxy for message input port.

*

* The caller must provide the handle to the initialized SceMi system,

* as well as the name of the transactor and port to which binding

* is requested.

*

* The 'binding' input is a callback function and context pointer tray.

* See the comments in scemitypes.h for struct SceMiMessageInPortBinding.

*/

SceMiMessageInPortProxy*

SceMiBindMessageInPort(

SceMi* mctHandle,

const char* transactorName,

const char* portName,

const SceMiMessageInPortBinding* binding,

SceMiEC* ec);

/*

* Create proxy for message output port.

*

* The caller must provide the handle to the initialized SceMi system,

* as well as the name of the transactor and port to which binding

* is requested.

*

* The 'binding' input is a callback function and context pointer tray.

* See the comments in scemitypes.h for struct SceMiMessageOutPortBind¬ing.

*/

SceMiMessageOutPortProxy*

SceMiBindMessageOutPort(

SceMi* mctHandle,

const char* transactorName,

const char* portName,

const SceMiMessageOutPortBinding* binding,

SceMiEC* ec);

/*

* Service arriving transactions from the portal.

* Messages enqueued by SceMiMessageOutPortProxy methods, or which are

* are from output transactions that pending dispatch to the

* SceMiMessageOutPortProxy callbacks, may not be handled until

* ServiceLoop() is called.  This function returns the # of output

* messages that were dispatched.

*

* The 'g' input is a pointer to a user-defined service function.

* If g is NULL, check for transfers to be performed and

* dispatch them returning immediately afterwards.  If g is

* non-NULL, enter into a loop of performing transfers and

* calling 'g'.  When 'g' returns 0 return from the loop.

* When 'g' is called, an indication of whether there is at

* least 1 message pending will be made with the 'pending' flag.

*

* The 'context' input is a user context object pointer.

* This pointer is uninterpreted by the SceMiServiceLoop()

* method and is passed on to the 'g' callback function.

*/

int

SceMiServiceLoop(

SceMi* mctHandle,

SceMiServiceLoopHandler g,

void* context,

SceMiEC* ec);

SceMiParameters*

SceMiParametersNew(

const char* paramsFile,

SceMiEC* ec);

unsigned int

SceMiParametersNumberOfObjects(

const SceMiParameters* parametersHandle,

const char* objectKind,

SceMiEC* ec);

int

SceMiParametersAttributeIntegerValue(

const SceMiParameters* parametersHandle,

const char* objectKind,

unsigned int index,

const char* attributeName,

SceMiEC* ec);

const char*

SceMiParametersAttributeStringValue(

const SceMiParameters* parametersHandle,

const char* objectKind,

unsigned int index,

const char* attributeName,

SceMiEC* ec);

void

SceMiParametersOverrideAttributeIntegerValue(

SceMiParameters* parametersHandle,

const char* objectKind,

unsigned int index,

const char* attributeName,

int value,

SceMiEC* ec);

void

SceMiParametersOverrideAttributeStringValue(

SceMiParameters* parametersHandle,

const char* objectKind,

unsigned int index,

const char* attributeName,

const char* value,

SceMiEC* ec);

/*

* SceMiMessageData initialization function.

* This is called to construct a new SceMiMessageData object.

*/

SceMiMessageData*

SceMiMessageDataNew(

const SceMiMessageInPortProxy* messageInPortProxyHandle,

SceMiEC* ec);

/*

* Destroy a SceMiMessageData object previously returned from

* SceMiMessageDataNew.

*/

void

SceMiMessageDataDelete(

SceMiMessageData* messageDataHandle);

/*

* Return size of message data array in 32 bit words.

*/

unsigned int

SceMiMessageDataWidthInBits(

const SceMiMessageData* messageDataHandle);

/*

* Return size of array in 32 bit words.

*/

unsigned int

SceMiMessageDataWidthInWords(

const SceMiMessageData* messageDataHandle);

/*

* Set value of message data word at given index.

*/

void

SceMiMessageDataSet(

SceMiMessageData* messageDataHandle,

unsigned int i,

SceMiU32 word,

SceMiEC* ec);

/*

* Set bit in message data word at given index.

*/

void

SceMiMessageDataSetBit(

SceMiMessageData* messageDataHandle,

unsigned int i,

int bit,

SceMiEC* ec);

/*

* Set bit range in message data word at given index.

*/

void SceMiMessageDataSetBitRange(

SceMiMessageData* messageDataHandle,

unsigned int i,

unsigned int range,

SceMiU32 bits,

SceMiEC *ec);

/*

* Return value of message data word at given index.

*/

SceMiU32

SceMiMessageDataGet(

const SceMiMessageData* messageDataHandle,

unsigned int i,

SceMiEC* ec);

/*

* Return value of bit in message data word at given index.

*/

int

SceMiMessageDataGetBit(

const SceMiMessageData* messageDataHandle,

unsigned int i,

SceMiEC* ec);

/*

* Return value of bit range in message data word at given index.

*/

SceMiU32

SceMiMessageDataGetBitRange(

const SceMiMessageData *messageDataHandle,

unsigned int i,

unsigned int range,

SceMiEC *ec);

/*

* Get cyclestamp.

*/

SceMiU64

SceMiMessageDataCycleStamp(

const SceMiMessageData* messageDataHandle);

/*

* This method sends a message with the specified payload to the

* transactor input port.  The data will transparently be delivered

* to the transactor as 1 or more chunks.

*/

void

SceMiMessageInPortProxySend(

SceMiMessageInPortProxy* messageInPortProxyHandle,

const SceMiMessageData* messageDataHandle,

SceMiEC* ec);

const char*

SceMiMessageInPortProxyTransactorName(

const SceMiMessageInPortProxy* messageInPortProxyHandle);

const char*

SceMiMessageInPortProxyPortName(

const SceMiMessageInPortProxy* messageInPortProxyHandle);

unsigned int

SceMiMessageInPortProxyPortWidth(

const SceMiMessageInPortProxy* messageInPortProxyHandle);

const char*

SceMiMessageOutPortProxyTransactorName(

const SceMiMessageOutPortProxy* messageOutPortProxyHandle);

const char*

SceMiMessageOutPortProxyPortName(

const SceMiMessageOutPortProxy* messageOutPortProxyHandle);

unsigned int

SceMiMessageOutPortProxyPortWidth(

const SceMiMessageOutPortProxy* messageOutPortProxyHandle);

#endif

  - Sample Header File for Basic Transaction Pipes C-Side API

As described in section Error: Reference source not found the infrastructure will provide the implementations of the actual built-in functions for the transaction pipes on both the C side and the HDL side. On the HDL side, the pipe functions and tasks in the SystemVerilog interface can be can be implementation supplied HDL definitions of built-in functions that perform the operations of the pipe inside. For example a pipe call can code that does PLI or DPI calls inside the tasks/functions in the SystemVerilog interface definitions. The way pipe functions are defined is up to the implementation but the must have the exact profiles shown in section Error: Reference source not found.

The following a precise listing of the include file that can be included by the C application to declare the API functions. The name of this file is scemi_pipes.h.

#ifndef _scemi_pipes_h

#define _scemi_pipes_h

#include "svdpi.h"

#ifdef __cplusplus

extern "C" {

#endif

//---------------------------------------------------------------------------

// scemi_pipe_c_handle()

//

// This function retreives an opaque handle representing a transaction

// input or output pipe given an HDL scope and a pipe ID.

//---------------------------------------------------------------------------

void *scemi_pipe_c_handle(      // return: pipe handle

const char *endpoint_path,  // input: path to HDL endpoint instance

int bytes_per_element,      // input: #bytes/element

svBit input_or_output );    // input: 1 for input pipe,

//---------------------------------------------------------------------------

// scemi_pipe_get_depth()

// scemi_pipe_set_depth()

//

// The scemi_pipe_get_depth() function returns the current depth of the pipe

// in terms of elements.

//

// The scemi_pipe_set_depth() establishes an internal buffer depth of

// the pipe in terms of elements.

//

// If either of these are the first call to a given pipe handle, the

// bytes_per_element establishes elmement size for the given pipe.

//---------------------------------------------------------------------------

int scemi_pipe_get_depth(

void *pipe_handle );     // input: pipe handle

void scemi_pipe_set_depth(

void *pipe_handle,       // input: pipe handle

int num_elements );      // input: capacity to be established for pipe

//---------------------------------------------------------------------------

// scemi_pipe_c_send()

//

// These is the basic blocking send function for a transaction input pipe.

// The passed in data is sent to the pipe. If necessary the calling thread

// is suspended until there is room in the pipe.

//

// The eom arg is a flag which is used for user specified end-of-message (eom)

// indication. It can be used for example to mark the end of a frame containing

// a sequence of transactions.

//

// scemi_pipe_c_receive()

//

// This is the basic blocking receive function for a transaction output pipe.

//

// The eom argument for this call is an output argument. It is set to the

// same settings of the flag passed on the send end of the pipe as described

// above. Thus is can be used by the caller to query whether the current

// read is one for which an eom was specified when the data was written on

// the send end.

//

// Both the send() and receive() calls are thread-aware. They can be

// easily implemented using a simple reference implementation that makes

// use of the non-blocking thread-neutral interface decribed below

// in conjunction with a selected threading system.

//---------------------------------------------------------------------------

void scemi_pipe_c_send(

void *pipe_handle,       // input: pipe handle

int num_elements,        // input: #elements to be written

const svBitVecVal *data, // input: data

svBit eom );             // input: end-of-message marker flag (and flush)

void scemi_pipe_c_receive(

void *pipe_handle,       //  input: pipe handle

int num_elements,        //  input: #elements to be read

int *num_elements_valid, // output: #elements that are valid

svBitVecVal *data,       // output: data

svBit *eom );            // output: end-of-message marker flag (and flush)

//---------------------------------------------------------------------------

// scemi_pipe_c_flush()

//

// Flush pipelined data.

//---------------------------------------------------------------------------

void scemi_pipe_c_flush(

void *pipe_handle );    // input: pipe handle

//---------------------------------------------------------------------------

// scemi_pipe_c_try_send()

//

// This is the basic non-blocking send function for a transaction input pipe.

// If there is room in the pipe for the indicated number of elements, the

// data is transferred to the pipe and a success status of 1 is returned.

// Otherwise, nothing is done with the data and a status of 0 is returned.

//

// This function is thread-neutral can can be used to create a reference

// implementation of the blocking send function (scemi_pipe_c_send)

// over a selected C-based threading environment.

//

// scemi_pipe_c_try_receive()

//

// This is the basic non-blocking receive function for a transaction output

// pipe. If the indicated number of elements exist in the pipe, the data is

// transferred out of the pipe and a success status of 1 is returned.

// Otherwise, the data in the pipe is left alone and a status of 0 is returned.

//

// This function is thread-neutral can can be used to create a reference

// implementation of the blocking receive function (scemi_pipe_c_receive)

// over a selected C-based threading environment.

//

//---------------------------------------------------------------------------

int scemi_pipe_c_try_send(

void *pipe_handle,       // input: pipe handle

int byte_offset,         // input: byte offset within data array

int num_elements,        // input: #elements to be written

const svBitVecVal *data, // input: data

svBit eom );             // input: end-of-message marker flag

int scemi_pipe_c_try_receive(

void *pipe_handle,       //  input: pipe handle

int byte_offset,         //  input: byte offset within data array

int num_elements,        //  input: #elements to be read

int *num_elements_valid, // output: #elements that are valid

svBitVecVal *data,       // output: data

svBit *eom );            // output: end-of-message marker flag

//---------------------------------------------------------------------------

// scemi_pipe_c_try_flush()

//---------------------------------------------------------------------------

int scemi_pipe_c_try_flush(

void *pipe_handle );     // input: pipe handle

//---------------------------------------------------------------------------

// scemi_pipe_c_can_send()

//

// This is function indicates if there is currently space in the pipe

// for the indicated number of elements meaning that the next call to

// scemi_pipe_c_send() will succeed without requiring a block.

//

// scemi_pipe_c_can_receive()

//

// This is function indicates if there is currently at least the indicated

// number of elements in the pipe meaning that the next call to

// scemi_pipe_c_receive() will succeed without requiring a block.

//

// For both of these calls, a return value 0 indicates that the operation

// cannot succeed, 1 indicates that it can succeed.

//---------------------------------------------------------------------------

int scemi_pipe_c_can_send(

void *pipe_handle,      // input: pipe handle

int num_elements );     // input: #elements to be written

int scemi_pipe_c_can_receive(

void *pipe_handle,      // input: pipe handle

int num_elements );     // input: #elements to be written

//---------------------------------------------------------------------------

// Notify callback support

//

typedef void (*scemi_pipe_notify_callback)(

void *context );       // input: C model context

void *scemi_pipe_get_notify_context(

void *pipe_handle );    // input: pipe handle

void scemi_pipe_set_notify_callback(

void *pipe_handle,      // input: pipe handle

scemi_pipe_notify_callback notify_callback,

// input: notify callback function

void *notify_context ); // input: notify context

int scemi_pipe_get_bytes_per_element( // return: bytes per element

void *pipe_handle );              // input: pipe handle

//---------------------------------------------------------------------------

// EOM auto-flush support

//

void scemi_pipe_set_eom_auto_flush(

void *pipe_handle,  // input: pipe handle

svBit enabled );    // input: enable/disable

#ifdef __cplusplus

} /* extern "C" */

#endif

#endif // _scemi_pipes_h