Requirements Document

API

for

Assertions

Version: 0.1
September 15, 2002

Document #

SVCC-AR090602-0.1

Accellera Copyright Notice

Committee Members

| Name | Company | Email |
| --- | --- | --- |
| Yatin Trivedi | ASIC Group | trivedi@pacbell.net |
| Francoise Martinolle | Cadence | fm@cadence.com |
| Stuart Swan | Cadence | stuart@cadence.com |
| Simon Davidmann | Co-Design | simond@co-design.com |
| Peter Flake | Co-Design | flake@co-design.com |
| Michael Rohleder | Motorola | Michael.Rohleder@motorola.com |
| Kevin Cameron | National Semiconductor | Kevin.Cameron@nsc.com |
| Bassam Tabbara | Novas | bassam@novas.com |
| Darrell Parham | Sun Microsystems | Darrell.Parham@sun.com |
| Joao Geada | Synopsys | Joao.Geada@synopsys.com |
| Ghassan Khoory | Synopsys | Ghassan.Khoory@synopsys.com |
| Andrzej Litwiniuk | Synopsys | Andrzej.Litwiniuk@synopsys.com |
| Alain Raynaud | Tensilica | alain@tensilica.com |
| Tarak Parikh | @HDL | tarak@athdl.com |
| Vassilios Gerousis | Infineon | Vassilios.Gerousis@Infineon.Com |
| Josef Derner | Mentor | josef_derner@mentorg.com |
| Karen Bartleson | Synopsys | Karen.Bartleson@synopsys.com |
| K Chen | Verplex | kchen@verplex.com |
| Michael McNamara | Verisity | mac@verisity.com |
|  | Xilinx | drm@xilinx.com |

Revision History

| Version | Date | Editor / Contributor | Comments |
| --- | --- | --- | --- |
| 0.0 | 8/14/02 | Joao Geada | Initial contribution from Synopsys |
| 0.1 | 9/12/02 | Yatin Trivedi | Created initial document brought in comments from email discussions |

Table of Contents

Revision History	4

List of Open Issues	6

1	Introduction	7

2	General Requirements	8

List of Closed Issues	11

Index	12

List of Open Issues

For now, I have copied comments from responses as they relate to specific requirements. Later, when the

List of open issues begin to form, we will use this as a quick reference for how we are doing.

Once we start closing a few items, I can move them to Closed Issues list at the end of the document (like an annex, and then remove (or separate) it eventually)

# Introduction

I will write some introduction part here regarding requirements.

# General Requirements

- Provide means (e.g. iterator) to obtain every assertion within a given instance

- Provide means to obtain every assertion associated with a given module   (sum of all assertions in all instances of the given module)

Bassam’s comments:

Let's just say - Provide means to obtain every assertion associated with a given module

It should not be "sum...", it's just things bound to module instead of instance (i.e. module ... { }) So it is really "intersection..." but let's not even say that. Just keep it for module bound, we have the instance bound in 1- above.

Joao’s response:

Good point. I agree.

- Provide means to get handle to an assertion by name

Bassam’s comments:

full name (meaning full_design_instance_name.assertion_name)

Joao’s response:

Yes, one way or another the name has to be a fully qualified name.

- Provide means to control assertions:

  - enable an assertion

  - disable an assertion

  - reset an assertion

Bassam’s comments:

We are still working in SV-AC on "reset"/"accept on"/"reject on"/.... so we

may need to add/remove a few things here.

Joao’s response:

Whether there are reject/accept constructs on in the language itself is one thing, but I really believe that the API should be able to exert some "low-level" control over the assertions. In this sense:

enable: allow the assertion to run as usual

disable: stop the assertion from running (& reset any internal state)

reset: behaves as disabled immediately followed by enable.

(in a sense: discard all current attempts, start new attempts on next clock)

- Provide means for applications to register callbacks for:

Bassam’s comments:

We probably need to classify callbacks (r/w/...)... Some callbacks are "read" (you cannot control assertion), whereas others are "write" (need a better word of course, but here you can enable/disable/reset/.... the assertion). Essentially the idea is the relationship between callbacks (of success/fail/...i.e. dynamic occurrences) and control.

Joao’s response:

I am not sure exactly what you mean here.

  - assertion success

  - assertion failure

  - assertion attempt

  - assertion being disabled

  - assertion being enabled

  - assertion being reset

Bassam’s comments:

Are these the only dynamic info ? What about things like assertion "local" vars (init/assign..), these objects will need to be available for access as well ... what else besides vars ?

Joao’s response:

Good catch.

There are really a couple of areas of both static and dynamic information for which tools will need access that I failed to include in the requirements:

+ static information (added to item 7)

+7.8: list of all HDL expressions referenced by an assertion (static drivers)

+10 - dynamic information on per attempt basis (new item 10)

+10.1: list of all HDL expressions actively driving the current state of the assertion

(on a per attempt basis: ie all expressions that were used by the assertion

on its last clock tick)

+10.2: values of all assertion "local" variables (this will depend, of course, on

the kind of local variables that will be present in the SV assertions)

+10.3: trace of expressions over time matched by the an specific attempt of an assertion

NOTE: there will have to be significant limitations on this requirement, eg

can only be provided for specific assertions and for a specific attempt and only

if this information is requested ahead of time.

Without limitations this requirement would kill all performance and capacity.

- Provide means to obtain per assertion static information:

  - assertion type (eg check/forbid/event/...)

  - assertion attributes (eg assume/guarantee/...)

Bassam’s comments:

Can we call this one "assertion modes" (or directive modes ...), I'd rather reserve attributes for 6.4

Joao’s response:

OK. We should use whatever terminology is consistent with the assertion standard.

  - source information (source file, line number, ...)

  - clocking signal/expression

  - instance in which assertion exists

  - name of assertion

  - module in which assertion declared

Bassam’s comments:

We need to generalize this one. "attributes", clocking signal is one attribute, there are others (weak/strong clock), others ? Of course, we should define how far we should go here ... what's is useful info for the scope of this API, and what's too much...

Joao’s response:

Is the clocking signal an attribute or a "driver" ?

I consider "attributes" to be things are generally informational hints to tools, and specifically to be things that should have no impact functionality/semantics. Both clocking and weak/strong clock have both functionality and semantics, so in my opinion these are "properties" of an assertion rather than "attributes".

But moving away from the terminology issue, I agree that there are probably a number of other "property/attribute/..." things that could be provided by the API _and_ that we should be careful on how many of these are made part of the API requirements.

- Provide means to obtain information about a specific success/failure:

  - time assertion attempt started

  - signal/expression where success/failure detected

List of Closed Issues

Index

assertion	9

attempt	9

attempt time	10

attributes	9

by name	8

controlling	8

disable	8

disabled	9

enable	8

enabled	9

failure	9

instance	9

module	9

name	9

reset	8, 9

static information	9

success	9

type	9

within a module	8

within an instance	8

attempt	9

attributes	9

clocking signal	9

disable assertion	8

disabled	9

enable assertion	8

enabled	9

failure	9

instance	9

module of declaration	9

name	9

register callback	8

reset	9

reset assertion	8

signal of assertion detection	10

source information	9

specific success	10

static information	9

success	9

time of assertion	10

type	9