System Verilog Assertion API

João Geada

Change Log

| Version | Date | Authors | Description |
| --- | --- | --- | --- |
| 0.1 | 11/25/2002 | Joao | First draft proposal for incorporating Synopsys assertions API donation into SV framework. |
| 0.2 | 12/03/2002 | João | Updated with comments/suggestions for sv-cc face to face meeting on 12/3/2002. Major changes: API changed to an extension of VPI, making all names consistent with naming scheme, additional static information APIs. Also synch-ed up terminology to current state of sv-ac (eg assertion  property) |

Table of Contents

1	Requirements	2

1.1	Naming conventions	2

2	Extensions to VPI enumerations	2

3	Static Information	3

3.1	Obtaining assertion handles	3

3.2	Obtaining static assertion information	4

3.2.1	Additional static information	5

4	Dynamic Information	5

4.1	Placing assertion “system” callbacks	5

4.2	Placing assertions callbacks	6

5	Control Functions	7

5.1	Assertion System control	7

5.2	Assertion control	7

# Requirements

To provide an API into the SV assertion capabilities providing sufficient capabilities to:

- enable user’s C code to react to assertion events

- enable 3rd party assertion “waveform” dumping tools to be written

- enable 3rd party assertion coverage tools to be written

- enable 3rd party assertion debug tools to be written

- The interface should also be readily extensible/adaptable so that it can easily be kept in sync with progress made by the sv-ac committee.

- In addition, this interface should not unnecessarily duplicate any existing PLI/VPI interfaces.

## Naming conventions

- All elements added by this interface will conform to the vpi interface naming conventions:

- all names will be prefixed by vpi

- type names will start with “vpi” followed by Capitalized words with no separators, eg vpiAssertCheck

- all function names will start with “vpi_” followed by all lowercase words separated by ‘_’, eg vpi_get_assert_info()

# Extensions to VPI enumerations

- Object types:

    - #define vpiProperty 130  /* temporal assertion */

    - #define vpiCheck     131 /* immediate (non-temporal) assertion */

  - Object properties:

    - #define vpiAssertProperty	132

    - #define vpiAssumeProperty	133

    - #define vpiRestrictProperty	134

    - #define vpiCoverProperty	135

  - Callbacks

    - Property related

      - #define cbPropertyStart       30

      - #define cbPropertySuccess  31

      - #define cbPropertyFailure   32

      - #define cbPropertyStep       33

      - #define cbPropertyDisable  34

      - #define cbPropertyEnable   35

      - #define cbPropertyReset     36

      - #define cbPropertyKill        37

    - “Property System” related

      - #define cbPropertySysInitialized 38

      - #define cbPropertySysStart          39

      - #define cbPropertySysStop          40

      - #define cbPropertySysEnd           41

      - #define cbPropertySysReset        42

  - Control constants

    - Property related

      - #define vpiPropertyDisable      136

      - #define vpiPropertyEnable       137

      - #define vpiPropertyReset         138

      - #define vpiPropertyKill            139

      - #define vpiPropertyEnableStep 140

      - #define vpiPropertyDisableStep 141

    - “Property System” related

      - #define vpiPropertySysStart  142

      - #define vpiPropertySysStop  143

      - #define vpiPropertySysEnd   144

      - #define vpiPropertySysReset 145

# Static Information

## Obtaining assertion handles

Extend the VPI module (actually, instance) iterator model to encompass assertions.

- Iterate over all properties in the design: use a NULL reference handle (ref) to vpi_iterate()
 	itr = vpi_iterate(vpiProperty, NULL);
 	while (assertion = vpi_scan(itr)) {
 		/* process property */
 	}

- Iterate over all properties in an instance: pass appropriate instance handle as reference handle to vpi_iterate()
 	itr = vpi_iterate(vpiProperty, instanceHandle);
 	while (assertion = vpi_scan(itr)) {
 		/* process property */
 	}

- Obtain assertion by name: extend vpi_handle_by_name to also search for assertion names in the appropriate scope(s)
vpiHandle = vpi_handle_by_name(assertName, scope)
Note that this operation only works for named assertions! Unnamed assertions cannot be found by name.

- NOTE: as with all vpi handles, assertion handles are handles to a specific instance of a specific assertion.

- NOTE: these iterators will return both temporal properties and immediate non-temporal checks.

## Obtaining static assertion information

The following information about an assertion is considered to be “static”:

- Assertion name

- Instance in which the assertion occurs

- Module definition containing the assertion

- Assertion type:

  - check

  - property

  - etc as necessary by assertion updates in sv-ac

- Assertion directive:

  - assert

  - assume

  - restrict

  - cover

  - etc as necessary by assertion updates in sv-ac

- Assertion source information
file, line and column where assertion defined

- Assertion clocking domain/expression

Static information can be obtained directly from an svaAssertID without requiring any assertion attempts to be started or completed.

typedef struct t_vpi_source_info {
 		PLI_BYTE* *fileName;

PLI_INT32 startLine;
PLI_INT32 startColumn;

PLI_INT32 endLine;
PLI_INT32 endColumn;

} s_vpi_source_info, *p_vpi_source_info;

typedef struct t_vpi_property_info {
 		PLI_BYTE8 *name;     /* name of property */

PLI_BYTE8 *instance; /* instance containing property */

PLI_BYTE8 *module;   /* module containing assertion */

PLI_BYTE8 *clock;    /* clocking expression */

PLI_INT32 type;      /* vpiProperty, vpiCheck */

PLI_INT32 directive; /* vpiAssume, … */

s_vpi_source_info sourceInfo;

} s_vpi_property_info, *p_vpi_property_info;

int vpi_get_property_info(assert_handle, p_vpi_property_info);

This call is used to obtain all the static information associated with an assertion. Note: a single call returns all the information for efficiency reasons, as most clients will require most of the data; for efficiency one roundtrip through the API is better than multiple roundtrips. The inputs are a valid handle to an assertion and a pointer to an existing s_vpi_property_info datastructure. On success the function returns TRUE and the s_vpi_property_info datastructure will be filled in as appropriate. On failure, the function returns FALSE and the contents of the property info datastructure are unpredictable.

### Additional static information

There are additional items that could be obtained statically from assertion, including:

- Structure of assertion

- Set of HDL variables used by assertion

- Set of HDL expressions used by assertion

No proposal for these items.

# Dynamic Information

## Placing assertion “system” callbacks

Use vpi_register_cb(), setting the cb_rtn element to the function to be invoked and the reason element of the s_cb_data structure to one of the following:

- cbPropertySysInitialized 
Occurs after system has initialized. No assertion specific actions can be performed until after this callback occurs. Note that the property system may initialize before or after cbStartOfSimulation

- cbPropertySysStart 
Assertion system has become active and will begin processing property attempts. Will always occur after cbPropertySysInitialized. Note that by default the property system will be “started” on simulation startup, but it is possible for a user to delay this with the appropriate use of property system control actions.

- cbPropertySysStop
Assertion system has been temporarily suspended. While stopped no property attempts will be processed and no property related callbacks will occur. The property system may be stopped and resumed an arbitrary number of times during a single simulation run.

- cbPropertySysEnd 
Occurs when all assertions have completed and no new attempts will start. Once this callback occurs no more property related callbacks will occur and property related actions will have no further effect. Typically occurs after the end of simulation.

- cbPropertySysReset
Occurs when the assertion system is reset, eg due to a system control action

The callback routine invoked follows the normal vpi callback prototype and is passed a s_cb_data containing the callback reason and any user data provided to the vpi_register_cb() call.

## Placing assertions callbacks

Use vpi_register_property_cb(), whose prototype is as follows:

vpiHandle vpi_register_property_cb(
   vpiHandle,			/* handle to property */
   PLI_INT32 event,		/* event for which callbacks needed */
   PLI_INT32 (*cb_rtn)(       /* callback function */
       PLI_INT32 event,
       vpiHandle property,

p_vpi_attempt_info info,

PLI_BYTE8 *userData),
   PLI_BYTE8 *user_data		/* user data to be supplied to cb */
);
typedef struct t_vpi_property_step_info {

/* contents to be defined from feedback from sv-ac (Bassam?) */

} s_vpi_property_step_info, *p_vpi_property_step_info;

typedef struct t_vpi_attempt_info {

union {

PLI_BYTE* failExpr;

s_vpi_property_step_info step;

} detail;
   s_vpi_time attemptTime,
} s_vpi_attempt_info, *p_vpi_attempt_info;

Where event is any of the following:

- cbPropertyStart
An assertion attempt has started. For most assertions one attempt will start each and every clock tick

- cbPropertySuccess
When an assertion attempt reaches a success state

- cbPropertyFailure
When an assertion attempt fails to reach a success state

- cbPropertyStep
progress of one “thread” along an attempt. Note that by default step callbacks are not enabled on any properties. Note also that step callbacks are enabled on a per-property/per-attempt basis, rather than on a per-property basis.

- cbPropertyDisable
Whenever the assertion is disabled (eg as a result of a control action)

- cbPropertyEnable
Whenever the assertion is enabled

- cbPropertyReset
Whenever the assertion is reset

- cbPropertyKill
When an attempt is killed (eg as a result of a control action)

These callbacks are specific to a given assertion; placing such a callback on one assertion will not cause the callback to trigger on an event occurring on a different assertion. If the callback is successfully placed a handle to the callback will be returned. This handle can be used to remove the callback via vpi_remove_cb(). If there were errors on placing the callback a NULL handle will be returned. As with all other calls, invoking this function with invalid arguments will have unpredictable effects.

Once the callback is placed the user-supplied function will be called each time the specified event occurs on the given property. The callback will continue to be called whenever the event occurs until the callback is removed.

The callback function will be supplied the following arguments:

the event that caused the callback,

the handle for the assertion,

a pointer to an attempt info structure

a reference to the user data supplied when the callback was placed.

The attempt info structure contains details relevant to the specific event that occurred:

- on disable, enable, reset and kill events the info field absent (a NULL pointer is given as the value of  info

- on start and success events, only the attempt time field is valid

- on a failure event, the attempt time and detail.failExpr are valid

- on a step callback, the attempt time and detail.step elements are valid.

NOTE: step callbacks are primarily intended to support property debugging and tracing applications, rather than general users.
NOTE: the actual contents and semantics of the step callbacks are still to be defined; pending information from sv-ac related to observable points in assertions (Bassam’s action item)

# Control Functions

## Assertion System control

Use vpi_control(), with one of the following operators and no other arguments:

- vpiPropertySysReset
discard all attempts in progress for all assertions and restore the entire assertion system to its initial state.

- vpiPropertySysStop
consider all attempts in progress as unterminated and disable any further assertions from being started.

- vpiPropertySysStart
restart the property system after it was stopped (eg due to vpiPropertySysStop). Once started, attempts will resume on all properties.

- vpiPropertySysEnd
discard all attempts in progress and disable any further assertions from starting.

## Assertion control

Use vpi_control() with one of the following operators:

- For all the following, the second argument must be a valid property handle.

  - vpiPropertyReset
discards all current attempts in progress for this assertion and resets this assertion to its initial state

  - vpiPropertyDisable
disables the starting of any new attempts for this assertion. Has no effect on any existing attempts. No effect if assertion already disabled. Note that by default all assertions are enabled.

  - vpiPropertyEnable
Enables starting new attempts for this assertion. No effect if assertion already enabled. No effect on any existing attempts.

- For all the following, the second argument must be a valid property handle and the third argument must be an attempt start time (as a pointer to a correctly initialized s_vpi_time structure)

  - vpiPropertyKill
Discards the given attempts but leaves assertion enabled and does not reset any state used by this assertion (eg past() sampling)

  - vpiPropertyEnableStep
Enables step callbacks to occur for this property attempt. No effect if stepping already enabled for this property+attempt. Note that by default stepping is disabled for all assertions.

  - vpiPropertyDisableStep
Disables step callbacks for this assertion. No effect if stepping not enabled or already disabled

João comment: these rules are meant to be consistent with the naming conventions used by PLI and VPI interfaces

To be merged to the contents of the “vpi_user.h”, described in 1364-2001, Annex G, pages 764-777
NOTE on final LRM merge, will have to verify that proposed number assignments remain unique

Exact types will have to be adjusted as per developments in  the sv-ac committee

Exact directives will have to be adjusted as per developments in  the sv-ac committee

Specific clocking domain info will have to be adjusted as per developments in  the sv-ac committee

NOTE can’t just use vpi_register_cb because the prototype of the callback function is different