CS4329-KS

End of Life Notification

- Please be advised that Cirrus Logic Inc., product CS4329-KS is nearing obsolescence.  The last time buy is August 15, 2001, and last time ship is February 15, 2002.

- The following table shows a comparison between the product that is nearing obsolescence and the recommended transition options.

|  | - CS4329-KS | - CS4390-KS | - CS4391-KS/KZ | - CS4392-KS/KZ |
| --- | --- | --- | --- | --- |
| - Maximum # of Bits | - 20 | - 24 | - 24 | - 24 |
| - Dynamic Range | - 106 dB | - 106 dB | - 108 dB | - 114 dB |
| - THD+N | - 97 dB | - 98 dB | - 94 dB | - 100 dB |
| - Power supplies | - +5 V | - +5 V | - +3 to 5 V | - +5 V |
| ###### Power consumption | - 210mW | - 210mW | - 85mW | - 85mW |
| - Sample rates | - 1 to 50 kHz | - 1 to 50 kHz | - 4 to 200 kHz | - 4 to 200 kHz |
| - Package size/type | - 20-pin SSOP | - 20-pin SSOP | - 20-pin SOIC/TSSOP | - 20-pin SOIC/TSSOP |
| - Pin compatible | - - | - Yes* | - No | - No |

- * The CS4390-KS does not support a 20-Bit right justified data format.

Contacting Cirrus Logic Support:

For a complete listing of Direct Sales, Distributor, and Sales Representative contacts, visit the Cirrus Logic web site at: http://www.cirrus.com/corporate/contacts/sales.cfm

or email: proaudio@crystal.com