Conversion Tables

Edited by joo@iWorld.net

The following table lists the subnet masks already converted using one octet for class A networks.

| Number of Subnets | Required number of bits | Subnet mask | Number of hosts Per subnet |
| --- | --- | --- | --- |
| 0 | 1 | Invalid | Invalid |
| 2 | 2 | 255.192.0.0 | 4,194,302 |
| 6 | 3 | 255.224.0.0 | 2,097,150 |
| 14 | 4 | 255.240.0.0 | 1,048,574 |
| 30 | 5 | 255.248.0.0 | 524,286 |
| 62 | 6 | 255.252.0.0 | 262,142 |
| 126 | 7 | 255.254.0.0 | 131,070 |
| 254 | 8 | 255.255.0.0 | 65,534 |

The following table lists the subnet masks already converted using one octet for class B networks.

| Number of subnets | Required number of bits | Subnet mask | Number of hosts Per subnet |
| --- | --- | --- | --- |
| 0 | 1 | Invalid | Invalid |
| 2 | 2 | 255.255.192.0 | 16,382 |
| 6 | 3 | 255.255.224.0 | 8,190 |
| 14 | 4 | 255.255.240.0 | 4,094 |
| 30 | 5 | 255.255.248.0 | 2,046 |
| 62 | 6 | 255.255.252.0 | 1,022 |
| 126 | 7 | 255.255.254.0 | 510 |
| 254 | 8 | 255.255.255.0 | 254 |

The following table lists the subnet masks already converted using one octet for class C networks.

| Number of subnets | Required number of bits | Subnet mask | Number of hosts Per subnet |
| --- | --- | --- | --- |
| Invalid | 1 | Invalid | Invalid |
| 1 ~ 2 | 2 | 255.255.255.192 | 62 |
| 3 ~ 6 | 3 | 255.255.255.224 | 30 |
| 7 ~ 14 | 4 | 255.255.255.240 | 14 |
| 15 ~ 30 | 5 | 255.255.255.248 | 6 |
| 31 ~ 62 | 6 | 255.255.255.252 | 2 |
| Invalid | 7 | Invalid | Invalid |
| Invalid | 8 | Invalid | Invalid |