# 1)  Frame-relay Technology

- LMI type default -> cisco

- Interface default -> multipoint

- Frame relay inverse-arp is on ( for point-to-point, there is no IARP)

- Split horizon checking  -> default 는 desable (같은 interface를 통하여 routing table이 update)

- Router는 LMI update를 통하여 frame relay switch로 부터 router가 필요로 하는 DLCI    number를 안다.

router는 IARP를 통하여 remote IP address를  알고 local DLCI를 mapping

Providers 1

2)  Configuring Hub and Spoke Frame Relay

Hub Router A

205.12.12.2

Switch A

Switch C

Switch B

Router B

205.12.12.3

Router C

205.12.12.4

- router 는 Frame relay switch로 부터 DLCI 를 안다

- router는 IARP를 통하여 remote IP address를 안다

Router A

Interface Ethernet 0

Ip address 205.25.25.1 255.255.255.0

!

interface Serial 0

ip address 205.12.12.2 255.255.255.0

encapsulation frame-relay

frame-relay lmi-type ansi

!

router rip

network 205.25.25.0

!

Router B

Interface Ethernet 0

Ip address 205.25.26.1 255.255.255.0

!

interface Serial 0

ip address 205.12.12.3 255.255.255.0

encapsulation frame-relay

frame-relay lmi-type cisco

!

router rip

network 205.25.26.0

Router C

Interface Ethernet 0

Ip address 205.25.27.1 255.255.255.0

!

interface Serial 0

ip address 205.12.12.4 255.255.255.0

encapsulation frame-relay

!

router rip

network 205.25.27.0

# 3)  Configuring Point-to-Point F/R subinterfaces

VC 구성 방식

Point-to-Point (전용선을 연결한 것과 같은 개념)

- 각 PVC 별로 별도 subnet 구성

- F/R subinterfaces support particially meshed F/R network.

- F/R subinterfaces는 multiple virtual interfaces를 지원하고 이것은 split horizon rules을 극복

For example

Point-to-point + dynamic address resolution

“interface dlci” -> IARP이용( dynamic address resolution)

Main Hub

DLCI 17

193.0.1.2

DLCI 16

193.0.2.2

Remote Site 1

DLCI 33

193.0.1.3

Remote Site 2

DLCI 33

193.0.2.3

Main hub

Interface Serial 0

No ip address

안주면

Multipoint

Encapsulation frame-relay

Frame-relay lmi-type ansi

!

interface Serial 0.1 point-to-point

ip address 193.0.1.2 255.255.255.0

frame-relay interface-dlci 17 -> dynamic mapping (dlci number를 가지고 상대방 IP 주소를 읽어온다.  상대편도 dynamic mapping이여야 한다)

!

Remote site 1

Interface Serial 0

Ip address 193.0.1.3 255.255.255.0

Encapsulation frame-relay

Frame-relay lmi-type cisco

!

Remote site 1

Interface Serial 0

Encapsulation frame-relay

No ip address

Interface s0.1 point-to-point

Ip address 193.0.1.3 255.255.255.0

Frame-relay interface-dlci 33

!

Remote Site 2

Interface Serial 0

Ip address 193.0.2.3 255.255.255.0

Encapsulation frame-relay ietf

Frame-relay lmi-type ansi

!

ietf option(이기종간 반드시 지정)

Encapsulation 형태지정

-  Standard encapsulation type은 ietf이고,PPP와 유사 (RFC 1490 spec.)

-  Default는 cisco -> 독자적인 encapsulation type

Cisco                    Ascend

## 4)  Configuring Dynamic and Static Mapping for Multipoint Subinterfaces

Static mapping removes the need for IARP requests

Multipoint : 여러 개의 PVC를 grouping 하여 1개 subnet구성

Main hub

193.0.1.2

DLCI 16

DLCI 17

Remote Site 2

193.0.1.4

DLCI 43

Remote Site 1

193.0.1.3

DLCI 33

Main hub

Interface Serial 0

Ip address 193.0.1.2 255.255.255.0

Encapsulation frame-relay

Frame-relay lmi-type ansi

Frame-relay map  193.0.1.3 16

Static Mapping

Frame-relay map  163.0.1.4 17 ietf

Remote Site 1

Interface Serial 0

No ip address

Encapsulation  frame-relay

Frame-relay lmi-type cisco

!

interface serial 0.1 multipoint

ip address 193.0.1.3 255.255.255.0

frame-relay map ip 193.0.1.2  33

Remote Site 2

Interface Serial 0

No ip address

Encapsulation frame-relay ietf

Frame-relay lmi-type  ansi

!

interface serial 0.1 multipoint

ip address 193.0.1.4 255.255.255.0

frame-relay map 193.0.1.2 43

# 5)  Another  Sample  Configuration

Router(config-if)#

Frame-relay map protocol  상대편 IP  local DLCI  [broadcast][ietf]

Protocol : network layer protocol{ip, ipx, appletalk etc}

Main Hub

S0.1 ip address 193.0.1.2 dlci 16

S0.2 ip address 193.0.2.2 dlci 17

Remote site 1

Serial ip address

193.0.1.3 dlci 32

Remote site 2

Serial ip address

193.0.2.3 dlci 33

Mail hub

!

interface s0

encapsulation frame-relay

no ip address

interface s0.1 point-to-point

ip address 193.0.1.2 255.255.255.0

frame-relay interface-dlci 16 -> local dlci --dynamic mapping

!

interface s0.2 point-to-point

ip address 193.0.2.2 255.255.255.0

frame-relay map ip 193.0.2.3 17 broadcast -> static mapping  상대편도 static mapping

반듯이 써주어야 한다(이유는 없음)

Router(config-if)#

Frame-relay map protocol  상대편 IP  local DLCI  [broadcast][ietf]

Protocol : network layer protocol{ip, ipx, appletalk etc}

Remote site 1

interface s0

encapsulation frame-relay

no ip address

interface s0.1 point-to-point

ip address 193. 0.1.3 255.255.255.0

frame-relay interface-dlci 32

!

Remote site 2

interface s0

encapsulation frame-relay

no ip address

interface s0.1 point-to-point

ip address 193. 0.2.3 255.255.255.0

frame-relay map ip 193.0.2.2 33 broadcast   static mapping

!