ping option중에 가는 pass와 오는 pass를 볼수 있는 option이있습니다.

c242#ping

Protocol [ip]:

Target IP address: 203.255.119.78(Transit GW)

Repeat count [5]:

Datagram size [100]:

Timeout in seconds [2]:

Extended commands [n]: y

Source address or interface: 203.255.117.242

Type of service [0]:

Set DF bit in IP header? [no]:

Validate reply data? [no]:

Data pattern [0xABCD]:

Loose, Strict, Record, Timestamp, Verbose[none]: r ---> Record option 선택

Number of hops [ 9 ]:

Loose, Strict, Record, Timestamp, Verbose[RV]:

Sweep range of sizes [n]:

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echoes to 203.255.119.78, timeout is 2 seconds:

Packet has IP options:  Total option bytes= 39, padded length=40

Record route: <*> 0.0.0.0 0.0.0.0 0.0.0.0 0.0.0.0

0.0.0.0 0.0.0.0 0.0.0.0 0.0.0.0 0.0.0.0

Reply to request 0 (1 ms).  Received packet has options

Total option bytes= 40, padded length=40

Record route: 203.255.117.242 203.255.119.125 203.255.119.77 203.255.119.78 ---> Transit GW까지 가는 Pass

203.255.119.78 203.255.119.126 203.255.117.124 203.255.117.242 <*> 0.0.0.0 ---> Transit GW에서 C242까지 오는 Pass

Server IP로 적용시에는 잘안되는데 라우터 IP에 적용시에는 잘되내요.